BASIC TIMER INSTRUCTIONS

Revised 12/16/08

Features:

- Can be used on a 6 Volt or 12 Volt system

- Can feed up to 5 different times per day

- Duration can be set from 1 second to 59 seconds – test function and each feed time can be set to a different duration if desired

- Timer is rated for 10 amps

Programming the Basic Timer:

Hook up timer to feeder and battery.  The display that comes up is the “Time of Day” display.  Note the hours, duration and minutes buttons.  Up arrow increases the value and down arrow decreases the value.

First, set current time of day.  Push clock button, clock icon at the top of the screen will flash.  Go up or down on the hours button until you reach the correct hour, am or pm.  Go up or down on the minutes button to get to the correct minute.  Next, go to the duration button and set the duration for your test function.  Push the clock button and your time of day is set and the duration for your test function is set.

Setting a feed time.  You can select any of the Feed Buttons, 1 thru 5, you do not have to start with 1 and stay in sequence.  Push Feed 1, a small feeder icon will appear on the screen blinking underneath Feed 1 button.  Use the hours button and set the correct hour that you want to feed, then using the minute button set the correct minute and then using the duration button set the desired duration.  [Remember on our feeders, a good rule of thumb is 5 seconds dispenses approximately 2 to 2 ½ lbs. of corn.]  Push Feed 1 button and the display goes back to the current time of day and the feeder icon quits blinking and remains solid on the screen.  Anytime a feeder icon is displayed solid on the screen, it means that a feed time is set and the feeder will feed at whatever time and duration has been entered in that feed time.

Repeat the above to set feed times in Feed 2, Feed 3, Feed 4 and Feed 5.

Test Feed.   With the current “time of day” displayed, push the test button, test on the timer display will blink, giving you a 4-5 second delay and then the feeder will feed for the number of seconds set.

To Delete a feed time.  Push the feed time you wish to delete.  Feeder icon will blink.  Push the Test/Clear button.  The display will return to the time of day screen and after a second or two the feeder icon on that deleted feed time will disappear.  That feed time is deleted.